~1: Common Network Issue Theory:

1. Five Common Hardware Issues Causing Network Problems

| No. | Hardware Issue | Symptom |
| --- | --- | --- |
| 1 | Damaged Ethernet cable | Network connection keeps disconnecting or shows “Network cable unplugged.” |
| 2 | Faulty network card (NIC) | Computer cannot detect any wired network connection. |
| 3 | Faulty switch port | Devices connected to one particular port cannot communicate with the network. |
| 4 | Router hardware failure | Multiple devices lose internet access, and router status lights may be abnormal. |
| 5 | Loose power/connection cable | Router, switch, or PC randomly turns off or loses network connectivity. |

2. Cisco Packet Tracer Troubleshooting

Network Setup

Devices:

- 3 PCs: PC0, PC1, PC2

- 1 Switch: Switch0

- 1 Router: Router0

- Ethernet cables

Topology:

PC0 ───┐

PC1 ───┼── Switch0 ─── Router0

PC2 ───┘

IP Configuration

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.2 | 255.255.255.0 |
| PC1 | 192.168.1.3 | 255.255.255.0 |
| PC2 | 192.168.1.4 | 255.255.255.0 |
| Router0 | 192.168.1.1 | 255.255.255.0 |

Troubleshooting Steps

- Open Cisco Packet Tracer.

- Create the topology with 3 PCs, 1 switch, and 1 router.

- Configure the IP addresses on all PCs and the router.

- Use the Ping command from PC0 to PC1:

ping 192.168.1.3

- Confirm that the ping works.

- Intentionally disconnect the Ethernet cable between PC1 and Switch0.

- Try the ping again:

ping 192.168.1.3

- The ping fails because PC1 is disconnected.

- Check the cable connection and the device/port status.

- Reconnect the cable and test the ping again.

- If the ping succeeds, PC1/cable connection was identified as the faulty part.

Result

The disconnected device/connection was identified by checking the topology, port status, and failed ping. After reconnecting it, communication was restored.

3. Troubleshooting Checklist for Slow Home Wi-Fi

- Check router power – Make sure the router is properly powered on.

- Check router status lights – Look for abnormal Internet/WAN or Wi-Fi indicators.

- Check cables – Ensure Ethernet/WAN cables are properly connected and not damaged.

- Restart the router – Power off the router, wait briefly, and turn it back on.

- Check Wi-Fi signal strength – Move closer to the router and check whether the speed improves.

- Check connected devices – Disconnect unnecessary devices that may be using bandwidth.

- Run a speed test – Compare the actual download/upload speed with the internet plan.

- Run a ping test – Use:

ping 8.8.8.8

Check for high latency or packet loss.

- Check device settings – Make sure Wi-Fi is enabled and the correct network is selected.

- Update network drivers – Check whether the laptop/PC's Wi-Fi driver needs an update.

- Check for background downloads – Pause Windows updates, cloud synchronization, or large downloads temporarily.

- Test another device – If all devices are slow, the problem is more likely with the router/ISP rather than one device.

4. Laptop Cannot Access Instagram and YouTube, but WhatsApp Web Works

The fact that WhatsApp Web works does not prove the internet connection is completely healthy. Instagram and YouTube could be affected by DNS, browser settings, firewall/filtering, or network configuration.

First 5 Troubleshooting Steps

1. Test other websites

- Open websites such as Google or Microsoft.

- Determine whether the problem affects only Instagram/YouTube or many websites.

2. Check the network connection

- Check the Wi-Fi icon and connection status.

- Disconnect and reconnect to Wi-Fi.

- Try another Wi-Fi network or mobile hotspot.

3. Test connectivity using Ping
Open Command Prompt and run:

ping google.com

ping youtube.com

Check whether the commands return replies or show packet loss/errors.

4. Check DNS settings

- Open the laptop's network adapter settings.

- Check whether DNS is configured correctly.

- Flush the DNS cache using:

ipconfig /flushdns

Then try accessing the websites again.

5. Check browser, firewall, and security settings

- Try another browser.

- Clear browser cache.

- Check whether a firewall, proxy, VPN, or security software is blocking Instagram or YouTube.

Conclusion:

If the sites work on another device/network, the problem is probably laptop configuration or software. If multiple devices cannot access them, investigate the router, DNS, ISP, or network configuration.

THE END……