~4-5: Weekly Practice and Troubleshooting Parctical:

Network Troubleshooting Practical

1. Test Connectivity Between Two PCs on Different Subnets

Objective

To test connectivity between two PCs located on different subnets and identify the difference between successful and failed communication.

Network Topology

PC0 ─── Switch0 ─── Router0 ─── Switch1 ─── PC1

IP Address Configuration

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC0 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC1 | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |
| Router0 G0/0 | 192.168.10.1 | 255.255.255.0 | — |
| Router0 G0/1 | 192.168.20.1 | 255.255.255.0 | — |

Router Configuration

Router> enable

Router# configure terminal

Router(config)# interface gigabitEthernet 0/0

Router(config-if)# ip address 192.168.10.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface gigabitEthernet 0/1

Router(config-if)# ip address 192.168.20.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# end

Successful Ping

From PC0, open Command Prompt and enter:

ping 192.168.20.10

Example output:

Pinging 192.168.20.10 with 32 bytes of data:

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Ping statistics:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

Failed Ping

To simulate a failure, change PC1's gateway incorrectly to:

192.168.20.254

Run the same command:

ping 192.168.20.10

Example:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss)

Diagnosis

The successful ping shows that routing between the two subnets is working.

The failed ping occurred because PC1 had an incorrect default gateway. PC1 could not correctly send traffic outside its local subnet.

Result

The correct default gateway was configured and connectivity between the two different subnets was successfully verified.

2. Incorrect Default Gateway Configuration

Objective

To identify and correct an incorrect default gateway so that a PC can communicate with an external web server simulation.

Example Network

PC0 ─── Switch ─── Router0 ─── Web Server

Correct Configuration

PC0:

IP Address:      192.168.10.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.1

Router:

G0/0: 192.168.10.1/24

Web Server:

IP Address: 192.168.30.10

Gateway:    192.168.30.1

Step 1 – Test Before Fix

From PC0:

ping 192.168.30.10

Example output:

Pinging 192.168.30.10 with 32 bytes of data:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss)

Step 2 – Check PC Configuration

Use:

ipconfig

Example:

IP Address........: 192.168.10.10

Subnet Mask.......: 255.255.255.0

Default Gateway...: 192.168.10.254

The gateway is incorrect.

The router's actual gateway is:

192.168.10.1

Step 3 – Correct the Gateway

In Packet Tracer:

PC0 → Desktop → IP Configuration

Change:

Default Gateway:

192.168.10.254

to:

192.168.10.1

Step 4 – Test Again

Run:

ping 192.168.30.10

Example output:

Reply from 192.168.30.10: bytes=32 time<1ms TTL=126

Reply from 192.168.30.10: bytes=32 time<1ms TTL=126

Reply from 192.168.30.10: bytes=32 time<1ms TTL=126

Reply from 192.168.30.10: bytes=32 time<1ms TTL=126

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

Result

The problem was caused by an incorrect default gateway. After changing the gateway to 192.168.10.1, the PC successfully communicated with the external web server.

3. Troubleshooting Access to Flipkart.com

Problem

A user reports that they cannot access www.flipkart.com from the simulated network.

Step 1 – Test Local Connectivity

First, test the PC's gateway:

ping 192.168.10.1

Example:

Reply from 192.168.10.1

Reply from 192.168.10.1

Reply from 192.168.10.1

Reply from 192.168.10.1

This confirms that the PC can communicate with its local router.

Step 2 – Test Remote Connectivity

Run:

ping 8.8.8.8

If this succeeds, the network has external connectivity.

Step 3 – Test Name Resolution

Run:

ping www.flipkart.com

Suppose the output is:

Ping request could not find host www.flipkart.com.

Please check the name and try again.

This suggests a possible DNS problem.

Step 4 – Use Traceroute

On Windows:

tracert www.flipkart.com

Example:

Tracing route to www.flipkart.com

1    1 ms    1 ms    1 ms    192.168.10.1

2    8 ms    7 ms    8 ms    10.10.10.1

3   15 ms   14 ms   15 ms    172.16.1.1

4   *       *       *        Request timed out.

Diagnosis

The first hop is the local router, so the PC-to-router connection is working.

If the route stops at an intermediate router, possible causes include:

- Incorrect routing table

- Missing static/default route

- Router interface failure

- ISP/upstream routing problem

- Firewall or ICMP filtering

A timeout alone does not always prove that the route is broken because some routers intentionally block traceroute responses.

Step 5 – Check Router Routing Table

On the router:

show ip route

Example:

C    192.168.10.0/24 is directly connected

C    192.168.20.0/24 is directly connected

If no default route exists, configure one if the topology requires it:

Router(config)# ip route 0.0.0.0 0.0.0.0 10.10.10.2

Result

The route was checked using tracert and the router's routing table. The missing route was identified and corrected. Connectivity was then tested again using ping and browser/web-server access.

4. Incorrect Static Route Between Two VLANs

Objective

To identify and correct an incorrect static route that prevents communication between two VLANs.

Example Topology

VLAN 10                  VLAN 20

192.168.10.0/24          192.168.20.0/24

|                         |

PC0                       PC1

|                         |

Router0 ─────────────── Router1

Problem

Router0 contains an incorrect static route:

ip route 192.168.20.0 255.255.255.0 10.0.0.3

But Router1's correct next-hop address is:

10.0.0.2

Step 1 – Check Routing Table

Run:

show ip route

Example:

S    192.168.20.0/24 [1/0] via 10.0.0.3

The route points to the wrong next hop.

Step 2 – Check Running Configuration

Run:

show running-config

Find:

ip route 192.168.20.0 255.255.255.0 10.0.0.3

Step 3 – Remove Incorrect Route

Router(config)# no ip route 192.168.20.0 255.255.255.0 10.0.0.3

Step 4 – Add Correct Route

Router(config)# ip route 192.168.20.0 255.255.255.0 10.0.0.2

Step 5 – Verify

Run:

show ip route

Expected result:

S    192.168.20.0/24 [1/0] via 10.0.0.2

Step 6 – Test Connectivity

From PC0:

ping 192.168.20.10

Example:

Reply from 192.168.20.10: bytes=32 time<1ms TTL=126

Reply from 192.168.20.10: bytes=32 time<1ms TTL=126

Reply from 192.168.20.10: bytes=32 time<1ms TTL=126

Reply from 192.168.20.10: bytes=32 time<1ms TTL=126

Result

The incorrect static route was identified using show ip route and show running-config. After replacing it with the correct route, communication between the VLANs was restored.

5. Troubleshooting WhatsApp Voice Calls Between Two Branches

Problem

Two company branches can exchange normal network traffic, but WhatsApp voice calls are not working between the branches.

Example Topology

Branch A                         Branch B

PC ─ Switch ─ Router A ─────── Router B ─ Switch ─ PC

|                  |

WAN Link           WAN Link

Step 1 – Check IP Configuration

On the PCs, verify:

IP Address

Subnet Mask

Default Gateway

DNS Server

Example:

Branch A

IP:      192.168.10.10

Gateway: 192.168.10.1

Branch B

IP:      192.168.20.10

Gateway: 192.168.20.1

Step 2 – Test Branch-to-Branch Connectivity

From Branch A:

ping 192.168.20.10

Example:

Reply from 192.168.20.10

Reply from 192.168.20.10

Reply from 192.168.20.10

Reply from 192.168.20.10

This shows that basic IP connectivity exists.

Step 3 – Check Routing

On both routers:

show ip route

Example problem:

Router A has:

S 192.168.20.0/24 via 10.0.0.2

but Router B has no return route to:

192.168.10.0/24

This creates an asymmetric/one-way routing problem.

Step 4 – Check Running Configuration

Run:

show running-config

Check:

- Static routes

- Interface configuration

- NAT configuration

- ACLs

- VPN configuration

- Default routes

Step 5 – Identify the Misconfiguration

Example:

Router B is missing:

ip route 192.168.10.0 255.255.255.0 10.0.0.1

Therefore, packets from Branch B do not have a proper route back to Branch A.

Step 6 – Correct the Configuration

On Router B:

RouterB(config)# ip route 192.168.10.0 255.255.255.0 10.0.0.1

Step 7 – Verify the Route

Run:

show ip route

Expected:

S    192.168.10.0/24 [1/0] via 10.0.0.1

Step 8 – Test Again

From Branch B:

ping 192.168.10.10

Example:

Reply from 192.168.10.10

Reply from 192.168.10.10

Reply from 192.168.10.10

Reply from 192.168.10.10

Important Note About WhatsApp Voice Calls

In a real network, successful ping does not guarantee that WhatsApp voice calling will work. Voice communication can also be affected by:

- NAT

- Firewall/ACL rules

- UDP traffic filtering

- VPN configuration

- DNS problems

- Internet connectivity

- Application/server restrictions

Therefore, after fixing routing, the relevant firewall/NAT/VPN configuration should also be checked.

Final Result

The problem was diagnosed as a missing return route on Router B. The required static route was added, the routing table was verified using show ip route, and bidirectional connectivity was confirmed using ping. Further NAT, ACL, or firewall checks would be required if voice traffic still failed.

Overall Conclusion:

The practical demonstrated a systematic troubleshooting process:

Check PC Configuration

↓

Test Local Gateway

↓

Test Remote Connectivity

↓

Use ping / tracert

↓

Check show ip route

↓

Check show running-config

↓

Identify Misconfiguration

↓

Correct Configuration

↓

Verify with show commands

↓

Test Connectivity Again

This method helps identify whether a network problem is caused by an incorrect IP address, default gateway, static route, routing failure, or another network configuration issue.

THE END………….